# Dissertation Proposal Form

PDE4445

Please fill out this form with the details for your project ideas. Refer to the appendix for clarification regarding the various terms.

Once finalized, you will also have to consider the ethical guidelines for the project. Typically, if your evaluation strategy involves human participants, you will be required to get prior ethical approval. Based on your understanding of the project and discussions with supervisor, please select the declaration that applies to you regarding ethical approval.

## Student Details:

| Student Name | Faseeh Mohammed |
| --- | --- |
| MISIS Number | M01088120 |

## Project Details:

| Proposed Title | 3D Perception and Motion Planning for PickerBot: Integrating ROS2, MoveIt2 and Intel RealSense RGBD into a Pick and Place Robotic System |
| --- | --- |
| Keywords | ROS2, MoveIt2, RGBD Perception, YOLOv8, 3D Object Detection, Point Cloud Processing, Pick-and-Place Robotics, Homography, Pose Estimation, EPSON VT6, TCP/IP Robot Integration, Industrial Automation |
| Problem Definition (Maximum 500 words) | Picker-Bot is an existing computer-vision-guided pick-and-place system built around an EPSON VT6-A901S 6-axis manipulator. The system occupies a partially validated state: its perception front-end is a live webcam feed processed by a YOLOv8 Oriented Bounding Box (OBB) model and demonstrated on real camera frames in the lab environment, and detections were confirmed on physical components. However, everything downstream of detection has never touched real hardware. Coordinate resolution relies on a manually collected homography grid that was never stress-tested against real-world conditions. All robot commands were sent to the EPSON RC8 virtual controller running inside the RC+ software environment, not to a physical arm. The system has therefore never executed a real pick operation. Bridging this gap between a lab-validated perception prototype and a fully deployed physical system is the central problem this dissertation addresses. First, the YOLOv8 model is undertrained and environmentally brittle. It was trained on only three component classes, entirely within a single lab setup with fixed lighting and a specific camera angle. No generalisation testing was performed across different lighting conditions, camera positions, or component orientations beyond those in the training set. On a real deployment surface where conditions vary, detection reliability is unknown. Second, the homography-based coordinate mapping is height-dependent and was never validated under realistic conditions. The 77-point calibration grid was collected once at a fixed camera height. On a real workspace where components tumble from a bucket and rest at arbitrary heights, this planar assumption breaks down entirely, producing incorrect pick coordinates for any component not lying flat at the calibration height. Third, there is no motion planning layer and no ROS2 integration. All robot commands in the current system are raw TCP strings sent directly to the RC8 controller. On a physical arm, this means no collision checking, no joint limit validation, no smooth trajectory interpolation, and no standardised sensor abstraction. Issuing unvalidated positional commands to a real manipulator in a cluttered workspace presents a safety and reliability risk that must be addressed before deployment. This dissertation addresses all three limitations as part of a single sim-to-real transition: retraining and hardening the YOLOv8 model for generalisation across real-world conditions, replacing the 2D homography pipeline with an RGBD depth-aware perception stack using an Intel RealSense D435/D455, introducing structured motion planning via ROS2 and MoveIt2, and validating the complete integrated system on the physical EPSON VT6-A901S arm for the first time. This work is one of two coordinated dissertations extending Picker-Bot. A parallel dissertation (Thesis 2 by Aman Mishra) addresses custom end-effector design and component pick verification using EPSON SPEL+ and ROS2. The ROS2 architecture adopted here is a deliberate shared substrate: this dissertation (Thesis 1) provides the perception and motion planning layer, while Thesis 2 builds the end-effector control and verification layer on top of it. A clearly defined ROS2 topic interface between the two subsystems is therefore a first-class deliverable of this work. |
| Research Question(s) | - To what extent does the existing YOLOv8-OBB model, trained on three component classes in a single controlled environment, generalise to real deployment conditions and what dataset augmentation or retraining strategy is sufficient to achieve reliable detection across varied lighting, camera positions, and surface backgrounds? - Can Intel RealSense RGBD camera combined with a ROS2 perception pipeline accurately estimate the 3D pose (position and orientation) of randomly placed and stacked microelectronic components on a work surface? - How can MoveIt2 motion planning be integrated with the EPSON VT6-A901S manipulator which lacks a native ROS2 driver to generate collision-aware pick trajectories from 3D pose estimates? - Does replacing the 2D homography approach with RGBD-based 3D pose estimation result in measurable improvements in pick accuracy, system reliability, and robustness to component stacking height variation? - How can the ROS2 perception pipeline be designed to expose a stable, well-defined interface that decouples the vision subsystem from the end-effector control subsystem? |
| Background Review (Maximum 500 words) | Pick-and-place robotics in industrial and laboratory settings has long relied on 2D vision systems. Classical approaches use fixed camera calibration matrices or planar homographies to map image coordinates to robot workspace coordinates. While adequate for flat, uniform-height workpieces, these methods are well-documented to fail under depth variation a known limitation addressed in the broader literature on bin-picking and unstructured manipulation (Collet et al., 2011; Buchholz et al., 2013). The emergence of affordable RGBD sensors, particularly the Intel RealSense D-series, has enabled point-cloud-based 6-DoF pose estimation at robot-compatible frame rates. Combined with deep learning detectors such as YOLOv8, recent systems have demonstrated reliable detection and 3D localisation of small objects in cluttered scenes (Ultralytics, 2023; Zhu et al., 2023). YOLOv8-OBB, already deployed in Picker-Bot, provides oriented bounding boxes that encode rotation angle a natural bridge towards full 3D pose estimation when paired with depth data. ROS2 (Robot Operating System 2) has become the dominant middleware for research and industrial robotics, providing a standardised publish-subscribe communication model, hardware abstraction, and a rich ecosystem of drivers and tools (Macenski et al., 2022). MoveIt2, the ROS2 motion planning framework, wraps planning libraries such as OMPL and trajectory execution managers, enabling collision-aware path planning with minimal configuration once a URDF robot model is defined (Coleman et al., 2014). A key open challenge for this dissertation is EPSON VT6 ROS2 integration. Unlike UR or KUKA arms, EPSON manipulators do not have a community-maintained ROS2 driver. Existing approaches for proprietary arms without native ROS2 support generally fall into two categories: (1) developing a hardware interface plugin that bridges ROS2 control commands to the vendor TCP/serial protocol, or (2) using MoveIt2 purely as an offline planner, with the resulting joint trajectory converted and forwarded to the vendor controller via the existing TCP link (Lentin Joseph, 2018). The latter approach preserves the working SPEL+ infrastructure and presents a viable path within the three-month timeline. The combination of RGBD perception, deep-learning-based detection, and ROS2/MoveIt2 motion planning represents the current state-of-the-art in flexible pick-and-place systems, and applying this stack to an existing working system with a documented 2D baseline provides a clear comparative evaluation opportunity. |
| Aims | This dissertation aims to execute the sim-to-real transition of Picker-Bot: taking a system whose perception front-end has been partially validated on real camera frames, but whose coordinate resolution and robot actuation have only ever run in simulation, and deploying it fully on a physical EPSON VT6-A901S manipulator. To achieve this, it aims to harden the YOLOv8 model for generalisation beyond its original training environment, replace the fragile 2D homography pipeline with a depth-aware RGBD perception stack, introduce collision-aware motion planning via ROS2 and MoveIt2, and validate end-to-end pick operations on real hardware under realistic workspace conditions. |
| Objectives (Bullet points) | • Audit and extend the existing YOLOv8-OBB training dataset beyond its original three-class, single-environment scope collecting additional samples under varied lighting, camera angles, and surface backgrounds and retrain or fine tune the model to improve generalisation to real deployment conditions. ● Set up a ROS2 (Humble/Iron) development environment and integrate the Intel RealSense D435/D455 using the realsense2_ros2 driver to stream aligned RGB and depth frames. • Extend the existing YOLOv8-OBB detection module to operate as a ROS2 node that publishes detected object poses as geometry_msgs/PoseArray on the ROS2 topic graph. • Implement a 3D pose estimation pipeline that back-projects YOLOv8 detection centroids through the RealSense depth map to recover (x, y, z) world coordinates, replacing the homography CSV approach. • Develop a ROS2-to-EPSON bridge node that translates MoveIt2 joint trajectory outputs into SPEL+ TCP commands compatible with the existing Main.prg receiver. • Conduct a quantitative evaluation comparing the new 3D pipeline against the existing 2D baseline across three metrics: 3D pose estimation accuracy, end-to-end pick success rate, and detection-to-execution cycle time. • Document the system architecture, integration decisions, and experimental results in a written dissertation meeting MSc standard. |
| Brief Methodology (Include Gantt chart) | Environment Setup (Weeks 1–2) Install ROS2 and configure the Intel RealSense D435/D455 with the realsense2_ros2 driver. Validate depth-RGB alignment and establish a calibrated camera-to-robot transform using a known calibration target. Set up the ROS2 workspace structure with the existing Picker-Bot codebase ported as a ROS2 package. 3D Perception Pipeline (Weeks 3–5) Wrap the existing YOLOv8-OBB model as a ROS2 node. For each detection, back-project the 2D centroid pixel through the registered RealSense depth frame to obtain a 3D point in the camera frame. Apply the camera-to-robot static transform to resolve robot-frame coordinates. Evaluate depth estimation accuracy against ground-truth measurements at known object heights. Motion Planning Integration (Weeks 6–9) Define the EPSON VT6-A901S in URDF, including joint limits and a simplified collision model of the workspace. Configure MoveIt2 with the OMPL planner. Develop the bridge node that subscribes to planned joint trajectories and converts them to timed SPEL+ TCP command sequences for the existing RC8 receiver. Validate motion execution on a small set of target poses. Evaluation and Write-up (Weeks 10–12) Run structured pick trials across three scenarios: flat single-layer components, stacked components, and a mixed scene. Record pose estimation error, pick success/failure, and end-to-end cycle time per trial. Compare results against the 2D baseline using the same component set. Write up findings. |
| Evaluation Strategy | Pose Estimation Accuracy — A set of 20 components will be placed at known ground-truth positions and heights (flat and stacked). The 3D world coordinates predicted by the RGBD pipeline will be compared against ground truth. Mean absolute error (MAE) in X, Y, and Z will be reported. The same positions will also be estimated using the 2D homography baseline (at calibrated height only) to establish a direct comparison. Pick Success Rate — A total of 50 pick trials will be conducted across flat-only, stacked, and mixed-height scenes. A pick is recorded as successful if the component is grasped and deposited in the collection tray without error. Success rate (%) will be compared between the 2D baseline (flat only) and the new 3D pipeline (all scenes). Cycle Time — End-to-end time from detection trigger to pick completion will be measured for both pipelines. The additional latency introduced by ROS2 communication and MoveIt2 planning will be quantified and compared against the direct TCP approach. |
| Ethical Declaration (i) or (ii) (Refer below) | (i) |
| Deliverables (Bullet points) | - A ROS2 package containing the RGBD perception node (YOLOv8 + depth back-projection), the MoveIt2 motion planning configuration, and the EPSON TCP bridge node. - Experimental evaluation dataset: pose estimation measurements, pick trial logs, and cycle time recordings for both the 2D baseline and the new 3D pipeline. - A defined and documented ROS2 topic interface likely a geometry_msgs/PoseArray or custom message type that publishes detected component poses for consumption by the end-effector control system (Thesis 2 of Pickerbot by Aman) - An interface specification document describing topic names, message types, coordinate frame conventions, and publish rates - Updated system documentation covering the ROS2 setup, launch files, and configuration parameters. - MSc dissertation |
| Resources Needed | Hardware • EPSON VT6-A901S 6-axis manipulator available in the lab • Intel RealSense D435 or D455 RGBD camera • Camera mounting bracket/stand • Component sample set microelectronic modules (same classes as trained in best.pt) for evaluation trials. Software • ROS2 (Humble or Iron LTS) on Ubuntu 22.04. • MoveIt2 • Intel RealSense SDK 2.0 and realsense2_ros2 driver — open source. • Ultralytics YOLOv8 • EPSON RC+ and RC8 controller software Access • Lab access during dissertation period for robot trials. |
| References | Buchholz, D., Winkelbach, S. and Wahl, F.M. (2013). RANSAC based 3D point cloud segmentation for bin-picking. Robotics and Autonomous Systems, 61(12), pp. 1359–1370. Coleman, D., Sucan, I., Chitta, S. and Correll, N. (2014). Reducing the barrier to entry of complex robotic software: a MoveIt! case study. Journal of Software Engineering for Robotics, 5(1), pp. 3–16. Collet, A., Martinez, M. and Srinivasa, S.S. (2011). The MOPED framework: object recognition and pose estimation for manipulation. International Journal of Robotics Research, 30(10), pp. 1284–1306. Intel Corporation (2023). Intel RealSense D400 Series Product Family Datasheet. Lentin Joseph (2018). Robot Operating System (ROS) for Absolute Beginners. Apress. Macenski, S., Foote, T., Gerkey, B., Lalancette, C. and Woodall, W. (2022). Robot Operating System 2: design, architecture, and uses in the wild. Science Robotics, 7(66). Ultralytics (2023). YOLOv8 Documentation. Zhu, X., Su, W., Lu, L., Li, B., Wang, X. and Dai, J. (2023). Deformable DETR: Deformable Transformers for End-to-End Object Detection. arXiv preprint arXiv:2010.04159. |

Signature:

Date: 29/05/2026

## Ethical Approval Guidelines

## Declaration A

- I have established that my study does not require additional human participation.

- I agree to re-apply for approval if the nature or goals of my project change.

## Declaration B

Project goals involve human participation:

- My study involves human participation through:

  - Observation

  - Questioning

- Participants will be selected without coercion.

- I will obtain written informed consent from each participant.

- I have arrangements in place for the protection of personal data.

- I agree to re-apply for approval if the nature or goals of my project change.

To be completed by the supervisor:

Supervisor Name:

Is the proposal acceptable? (YES/NO):

If not, please provide feedback:

Signature:

Date:

## Appendix: Explanation of Typical Terms Used in Research Proposals

- Title: Write a title which briefly describes the research problem and your approach to it.

- Keywords: To give a clear and concise description of the scope and nature of the report, such as the main variables to be considered.

- Problem definition: Correctly defining the problem is the crucial first step in the research process. If the research problem is defined incorrectly, the research objectives will also be wrong. Problems must be stated in terms of underlying causes – they must be structured in a way in which they can lead to a solution. It is critical that the statement be useful for development and evaluation of potential solutions.

- Research Question: Indicate what you want to know most and first out of your research. These are typically written in a ‘question’ format, for example, ‘Does a robotic exoskeleton provide an efficient solution to physical rehabilitation of the upper arm for stroke patients?’ Multiple research questions should be put in a list. Questions should not be answered by a simple YES or NO, they must generate a discussion.

- Background Review: Explain the technical/discipline area you will be working in, the problem area that you will be addressing in your research, and where you would locate your intended work in relation to previous researchers or existing solutions.

- Aims: They represent the changes you hope to achieve as a result of your work.

- Objectives: These are the activities you undertake and the methods you propose to bring these changes about. They should be in a bulleted list, and more specific than the aims. In fact, this list should be a way of breaking down the aims into more achievable tasks. Anywhere between 4 – 7 objectives are usually considered appropriate for a project of this scale.

- Methodology: A brief description of the steps to be followed to achieve the objectives that have been set out. This section should describe the development plan.

- Gantt chart: A Gantt chart is a horizontal bar chart used in project management to visually represent a project plan over time.

- Evaluation Strategy: Methods and techniques used in evaluating the project. There are two basic approaches, quantitative and qualitative. Quantitative involves the generation of data, through experiments or simulation, in quantitative form, which can then be analysed. Qualitative is concerned with subjective assessment of opinions, behaviour, impressions via, for example, interviews.

- Deliverables: Defining your intended outcomes (be as specific as possible). It is part of good project planning. Deliverables are linked to your aims.

- Resource Needed: Information on the hardware, software, and research resources you will be using during your research. Be specific – Give the name of any software and hardware, rather than stating something generic.